DC-USB

6-34V 10A, Intelligent DC-DC converter with USB interface

Quick Installation Guide

Version 1.0c

P/N DC-USB

Introduction

The DC-USB is a small yet powerful DC-DC power supply designed to

power a wide variety of devices. This DC-DC has a r ange of intelligent

functions not found in any tradition USB converters. Features include USB

interface, programmable output voltage and scripting as well as automotive

modes.

The DC-USB dev ice is able to send ON/OFF ‘pulse signals’ to

motherboards based on voltage levels or Ignition sensing, making it an ideal

device for automotive or battery powered installations.

This unit has a wide input range (6-34V) and it can prov ide a tightly regulated

output ranging from 6 to 24V (default set to 12V).

Mini-Box.com   ATX DC-DC Converter Series

Quick installation Instru ctions

Fig 1.0, DC-USB layout

CONF IG:

Configuration jumpers for Voltage, Mode and Timings.

Left m ini-FIT connector

:    Power input, V(in), GND, Ignition.

Right mini-FIT connector

: Power output V(out), GND.

PSW:

Soft ON/OFF control for motherboard. Connect this to motherboard

ON/OFF pins if you want the motherboard to be controlled by the unit.

USB:

mini-USB type B jack. Connect this to a PC to access advanced

settings.

STATE

: State LED

Vaux:

Provides unregulated switched input, to be used in automotive modes

to power various peripherals. Also, in automotiv e mode, Vaux can be used as

‘thump’. Vaux should be closer to V(In) in this case.

Basic Op eration

For basic operation, you would need to connect a power source to V(In).

V(IN) is on the left side of the board, near the 10A input fuse. Polarity is

marked on the PCB (GND, V(In) and Ignition).  O n the cable harness, GND is

black, V(in) is red and Ignition is white. NOT E: Ignition is not needed for basic

operation.

Without any further settings, V(out) will generate 12V regulated. V(out) is

located on the right side of the PCB, near the USB connector. On the cable

harness, Yellow is positive and GND is negative.

Configuration jumpers

The configuration header (marked CONFIG) is the most important header in

this board. It is divided in 3 sections:

Tim ing Schem es

Main mod e

Ou tput voltage

DC-USB Quick Installation G uide  Page 2

Mini-Box.com   ATX DC-DC Converter Series

Main mode.

This header section sets one of the 3 main mode of operation:

DUMB mode, Automotive mode and the SCRIPT mode.

Output voltage.

This header section sets the output voltage of the unit.

Timing Schemes.

This header section sets the OFFDELAY and HARDOFF

timers, only av ailable in the Automotive mode.

Configuration, voltage settings

By default, the DC-USB module provides regulated 12V output. Should

you need other voltage levels, you can change output voltages by setting

jumpers 3, 4 and 5, see table 1.

After making a jumper change, the DC-USB unit needs to be power cycled

in order for the new setting to take effect.

NOTE:

These settings are factor y

defaults. They can all be changed with the DC-USB software. F or this, see

Advanced USB Configuration manual.

Configuration, Mod e of Op eration

DC-USB has 4 modes of oper ation. This modes c an be changed via

jumpers.

1) DUMB mode

. The units acts as a regular  DC-DC converter. Only V(In) and

V(out) and GND terminal are required. Unit will conv ert any input from 6-34V

to a fixed voltage. Default voltage is set to 12V.

2) AUTOMOTIVE MODE.

In this mode, the unit acts as as Intelligent DC-DC

converter that is aware of Ignition state. In this mode the unit reads the

Ignition terminal and based on Ignition status the unit sends ON/OFF  pulses

to a motherboard in order to start or stop. In this mode, two variables can be

set: OF FDELAY and HARDOFF . See Default Timing Schemes for more

information.

DC-USB Quick Installation G uide  Page 3

Mini-Box.com   ATX DC-DC Converter Series

3) SCRIPT MODE.

T his is an adv anced mode where unit can be scripted to

perform various tasks based on user scripts.  Please refer to Advanced USB

programming manual for more details.

4) UPS MODE.

Module is a DC UPS (Uninterruptible Power Source) module.

Default Timing Schem es

When unit is set to oper ate in the Automotiv e Mode, 8 timing settings are

available. These are combinations of OFFDELAY and HARDOF F timers.

NOTE: These settings are factory defaults. They can all be changed with the

DC-USB software. F or this, see Adv anced USB Configuration manual.

These settings work only when the Ignition wire is used.

OFFDELAY

is the amount of time the unit waits until it sends an ON/OFF

pulse the motherboard’s ON/OFF pins after Ignition has been turned off.

HARDOFF

is the amount of time the unit will still provide power after the

ON/OFF pulse has been sent to the motherboard. If you have a battery

sensitive application, set the HARDOFF to 1 minute to avoid battery drain.

While in HARDOF F, the unit carefully monitors the battery and if battery

voltage goes under 11.2V, power will be cut off in order to prevent battery

drainage.

Mini-Box.com   ATX DC-DC Converter Series

Electrical and Environmental Specifications

Minimum Input Operating  voltage  6V

Maximum input Operating voltage  Electronic shut down at 34V (clamping will occur

at 34-36V)

Deep-Discha rge shutdown threshold  11.2V

Input current limit (fuse protected)  10A (10A mini-blade fuse)

Max Output Powe r  100watts (lim ited by 10A input fuse) *

Regulation accuracy  2.5%

Operating temperature  -40 to +85 degre es Celsius

Storage tempe rature  -55  to +85 degrees Celsius

MTBF  >100,000 hrs @ 65C body temp.

Effic iency (Input 9-16V)  >95% (output = 12V 2A)

PCB size  135mx37mm

Input, output connectors  Right angle, Mini-FIT JR 4 pin

*NOTE: At output power greater than 40watts or if unit temperature exceeds

65C, forced air ventilation will be required in order to prevent unit from

excessive thermal stress for long period of times.

12V outpu t m ax current charts

Input (V)  12V rail curre nt  Input (V)   12V ra il curre nt

6V  4A  11V  8A

7V  5A  12V  8A

8V  6A  14V  8A (10A peak)

9V  7A  14-18V  8A (10A peak)

10V  8A  20-26V  8A

Operating environment

: Temperature: -20 to 65 degree centigrade.

Relative Humidity: 10 to 90 percent, non-condensing.

Efficien cy, MTBF:

MTBF >100K hours at PSU (temp) < 65 Celsius.

NOTE: All solid polymer capacitor design, rated >50K hours at 85C or 500K

hours at 65C.

Shipp ing  and sto rage:

Temperature -40 to +85 degree centigr ade.

Relative humidity 5 to 95 percent, non-condensing.

Warranty

1 Year Limited W arranty statement.  Warranty is void if maintenance or

calibration is performed by end-user or by use in conjunction with power

modules not prov ided by mini-box.com.